| #### Ref. COM 08 | #### Task/Activity: Use of Pressure Washers |  |  |  | #### Assessment date: |
| --- | --- | --- | --- | --- | --- |
| #### Unit No: | #### Unit name/location: |  |  |  | #### Assessed by: |
| Hazard |  |  | Safety Risk | #### Safe System of Work / Control Measures |  |
| Electrical hazards Spillages of water/ wet surfaces Trailing hoses and cables High pressure water jets Pressure vessels Manual handling Cleaning products Pressure Washers used in this site are: |  |  | Electric shock Slips, trips and falls Bruising, flesh wounds Explosion Back and muscle strain Irritation to skin and eyes | - Pressure washers only to be used and cleaned by trained employees. - Manufacturer's user instructions to be followed where these are available - Pressure washers to be maintained by competent persons and in accordance with manufacturer's guidance where available. Pressure washers to be inspected in accordance with a written scheme of examination by a competent person at appropriate intervals. - Suitable Personal Protective Equipment to be worn in accordance with manufacturer’s user instructions. This must include eye protection. Use at all times, if damaged or mislaid, report it to your manager. - Pressure washers always to be used in conjunction with a residual current device (RCD). - Reduce the length of trailing cables and minimise the use of extension leads by using the nearest electrical socket outlet. Cables to be kept away from the area being cleaned and behind the user. - Ensure no strain is placed on the electrical cable and connections. - Do not point the water nozzle at any part of the body or towards any other person. - Do not direct the water nozzle close to electrical equipment or sockets. - Wherever practicable, pressure washers to be used in an area that will be subject to the least possible pedestrian movement. Erect suitably located warning signs/barriers as appropriate to warn or prevent unauthorised access when pressure washing is in progress. Ensure the area is appropriately dry before removing signage. - Do not leave the machine unattended in a public area. - Disconnect from the mains electrical supply before assembling, cleaning or maintaining the pressure washer. - Implement the Safe System of Work/Control Measures in the Electrical Safety Risk Assessment, Ref MAN 05 including annual PAT test and the safety precautions in the Manual Handling Checklist and COSHH Product Assessments. |  |
| Who may be affected by the task/activity? (Tick all applicable boxes) |  |  |  |  |  |
| Compass employees/Agency staff |  |  |  |  |  |
| Customers / Client staff |  |  |  |  |  |
| Visitors / Members of the Public |  |  |  |  |  |
| Maintenance personnel |  |  |  |  |  |
| Delivery personnel |  |  |  |  |  |
| Other (specify below) |  |  |  |  |  |
| IMPORTANT - This risk assessment should be reviewed every 3 years, or whenever there is a significant change in the task or activity and following any accident or incident involving this task or activity. This risk assessment must be retained for a period of 6 years. |  |  |  |  |  |