| TASK SPECIFIC RISK ASSESSMENT |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Unit Address: |  |  |  |  |  |  | Tel: Fax: | Unit No: |  |  | Assessment No. |  |  |
|  |  |  |  |  |  |  |  | Assessment Date: |  |  |  |  |  |
|  |  |  |  |  |  |  |  | Review Date: |  |  |  |  |  |
| Unit Manager: |  |  |  |  | Line Manager: |  |  | Assessment completed by: (Print Name & Job Title): |  |  |  |  |  |
|  |  |  |  |  |  |  |  | Signature: |  |  |  |  |  |
| Copies to: /X |  | Operator  Team Leader  Client BDM Insurance Dept. File  |  |  |  |  |  |  |  |  |  |  |  |
| A. Activity and or environment(s) assessed |  |  |  |  |  |  |  |  |  |  |  |  |  |
| PUMPING OUT TUNNEL SUMPS |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ## General Comments: |  |  |  |  |  |  |  |  |  |  |  |  |  |
| B. Persons Exposed  if present. If not applicable – leave blank |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Company Employees |  |  |  |  | Client Employees / Visitors |  |  |  | Contractors |  |  |  |
|  | New / Expectant Mums1 |  |  |  |  | New Starters |  |  |  | Delivery Personnel |  |  |  |
|  | Elderly (over 65 years) |  |  |  |  | Young Workers2 |  |  |  | Children2 |  |  |  |
|  | People outside of the site |  |  |  |  | Agency Staff |  |  |  | Special Needs |  |  |  |
|  | Night Workers3 |  |  |  |  | Lone Workers |  |  |  | Other (Detail): |  |  |  |

- Separate ‘New & Expectant Mothers Risk Assessment’ to be completed for each person.

- Separate ‘Children & Young Persons Risk Assessment’ to be completed for each person.

- Separate ‘Medical Questionnaire’ to be completed for each night worker. Under the ‘Working Time Regs’ each night worker is entitled to a free health assessment on request. Initially, this involves completion of a revised copy of the Company’s  ‘Medical Questionnaire’.

| C. Hazards & Risks: Insert the following into the boxes below: H = High Risk-Likely to occur and serious injuries probable, M = Medium Risk– Could occur and injuries could possibly be serious, L = Low Risk-Unlikely to occur and if it did only minor injury likely. Leave blank if not applicable |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| #### Working Practices |  |  | #### Physical Injury |  |  | #### Environment |  |  |
|  |  | Lone Worker | 21 |  | Assault / Violence | 42. |  | Working Outside |
|  | M | Areas Difficult to Access | 22 | M | Portable Equipment Hazards | 43. | L | Confined Spaces |
|  |  | Hot Oil | 23 |  | Hand Tools | 44. | M | Access to First - Aid |
|  | L | Spillage’s / Wet Floors | 24 |  | Large Equipment Hazards | 45. |  | Work overhead |
|  |  | Non routine practices | 25 |  | Moving Machine Parts | 46. |  | Excavations |
|  | L | Contamination Areas | 26. | M | Slips, Trips, Falls | 47. |  | Extremes of temperature |
|  |  |  | 27. | M | Falls from Heights | 48. | L | Ventilation |
|  |  |  | 28. |  | Sharps | 49. |  |  |
| ###### Physical Agents |  |  | 29. |  | Transportation Hazards | Training (Tick) |  |  |
|  | L | Electricity | 30. | M | Falling Objects | 50. |  | Food Hygiene |
|  |  | Fire | 31. |  |  | 51. |  | Basic Health & Safety |
|  |  | Hot/Cold Objects | 32. |  |  | 52. |  | ## Display Screen Equipment |
|  |  | Noise/Vibration | ###### Hazardous Substances |  |  | 53. |  | Cleaning/COSHH/Manual H. |
|  |  | Animals | 33. |  | Contact with Chemicals | 54. |  |  |
|  |  | Arc Flash/ UV Light | 34. |  | Bacteria etc. (e.g. Legionnaires) | 55. |  |  |
|  |  | Explosion | 35. | L | Dust/ Fumes/ Gases/ Vapours | 56. |  |  |
|  |  | Pressure Systems / Gases | 36. | L | Vermin/Weils Disease | 57. |  |  |
|  |  |  | 37. |  | Zoonotic Diseases | ###### Other Issues (Tick) |  |  |
| ###### Manual Handling |  |  | 38. |  |  | 58. |  | Documentation /Signage |
|  | M | Handling of Loads | 39. |  |  | 59. |  | PPE |
|  |  | Use of Lifting Aids | 40. |  |  | 60. |  | BA Sava Set |
|  |  |  | 41. |  |  | 61. |  |  |

| (d) Significant Risks Identified (High, Medium or Low Risk) |  |  | (e) Control Measures to be Applied | (f) Responsibilities |  |
| --- | --- | --- | --- | --- | --- |
| No | R |  |  | By Who | When |
| 2 4 6 | M L L | Working Practices Areas Difficult to Access Spillage’s/Wet Floors Contamination Areas | By the very nature of all site tunnels, access is predominantly via vertical ladders after gaining entry through a locked hatch. The vertical ladders have safety surrounds at the initial point to the ladder from the floor above. Each worker should check his shoes to insure they will not slip on the ladder rungs. After entry is made to the tunnel, access to the sump should be assessed to insure the pump or hose can be inserted safely. If safety is not assured stop the job and consult your supervisor. Water spillage’s need to be contained and cleared immediately. Wet floor signs need to be displayed whilst the spillage is being cleared and whilst the floor is being wet cleaned. In the tunnels a spillage will usually seek the lowest point which could be the sump you will be pumping out. Many of the systems on site contain contaminated liquor. Therefore you should never assume the liquid you are to pump is clean. A sample should be taken and assessed before commencing work. If the sample is contaminated guidance must be sought from Health Physics before proceeding. Follow the guidance given by Health Physics and or any radiological documents provided. A Eurest radworker can be put in charge of the work party if it is deemed necessary. | Super- Visor/ Worker Worker ##### Super- Visor/ Worker | Each Time Each Time Each Time |

| (d) Significant Risks Identified (High, Medium or Low Risk) |  |  | (e) Control Measures to be Applied | (f) Responsibilities |  |
| --- | --- | --- | --- | --- | --- |
| No. | R |  |  | By Who | When |
| 9 | L | Physical Agents Electricity | Normal precautions need to be taken to avoid contact with ‘live’ parts. Electric leads are approved and with current PAT test dates. Keep electric plugs clear of water. Run electric leads in cable trays where possible and minimise or eliminate tripping hazards. | Super- Visor/ Worker | Each Time |
| 18 | M | Manual Handling Handling of Loads | The most significant loads are likely to be involved in the movement of the pumps and holding drums for the liquid. No equipment weighing more than 25kg will be taken into the tunnels without the use of an approved lifting rig. An additional assessment must be done for any tunnel work that requires heavier equipment being mobilised. | Super- Visor/ Worker | Each Time |
| 22 | M | Physical Injury Portable Equipment Hazards | All electrical equipment must be PAT tested and be within its date for re-testing. Users must do a quick visual check of the condition of the flex/plug etc. prior to use. Before changing disposable parts on equipment, the equipment must be disconnected from the electrical supply. In the case of tunnel work electrical outlets are not generally available in the tunnels. Establish and maintain good two-way communications between the connection/disconnection point and the point of work. | Super- Visor/ Worker | Each Time |

| (d) Significant Risks Identified (High, Medium or Low Risk) |  |  | (e) Control Measures to be Applied | (f) Responsibilities |  |
| --- | --- | --- | --- | --- | --- |
| No. | R |  |  | By Who | When |
| 26 27 30 | M M M | Physical Injury (continued) Slips, Trips, Falls Falls from Heights Falling Objects | The contents of sumps could be oils and muddy and either of these adds to the possible slippery conditions in the tunnels. Clear up any spills and always be alert to leaks and spills. As stated earlier, watch out for the trip hazard associated with trailing leads and the hose connected to the pump. The only route available for getting the pump and other required equipment into position in the tunnels is via the vertical ladder aperture. Exercise extreme care when lowering and raising the equipment in and out of the tunnels. No more than 25kg loads are authorised for this method of entry. A fully trained rigger/slinger must control the movements. No worker is allowed to work under the load being lowered or raised. The equipment being taken in and removed from the tunnels will be lowered and raised on rope lanyards. The rigger/slinger doing the job will ensure all non-essential members of the working party remain clear of the area when equipment is being mobilised and demobilised. The worker at the top should insure the area around the opening is free of all loose objects that could be accidentally knocked into the opening. All members of the work party must wear hard hats. | Super- visor/ Worker Super- Visor/ Worker Super- Visor/ Worker | Each Time Each Time Each Time |

| (d) Significant Risks Identified (High, Medium or Low Risk) |  |  | (e) Control Measures to be Applied | (f) Responsibilities |  |
| --- | --- | --- | --- | --- | --- |
| No. | R |  |  | By Who | When |
| 35 36 | L L | #### Hazardous Substances Dust/Fumes/Gases/Vapours Vermin/Weils Disease | Any work around sumps involves the possibility of encountering hazardous gases and vapours. Entry into and work in tunnels must be done with an alarming multigas meter. Dead rats have occasionally been found in tunnels on site. Use care when working around sumps. Wear protective gloves and always wash your hands following work sessions around the sumps. Each member of Plant staff should be trained in safe working practices for the prevention of Weils disease. | Worker Worker | Each Time Each Time |
| 43 44 48 | L M L | #### Environment Confined Spaces Access to First Aid Ventilation | Personnel must be suitably trained, experienced and competent to carry out work in a confined space. Check ‘Permit to Work/ Safety Document’ carefully to ensure an oxygen monitor and a 15-minute emergency oxygen supply (‘Sava-set’) is used if required. Due to the remote location familiarise yourselves with the location of the nearest phone. Always inform SSCR on (Site Specific) prior to entry of the names of all members of the work party. The provision and use of a multigas meter will preclude any vent- illation difficulties. If oxygen supplies decrease leave the tunnel immediately and report to your supervisor and Health Physics | Worker Super- Visor/ Worker Worker | Each Time Each Time Each Time |

| SAFE SYSTEM OF WORK (‘S.S.W.’) This sheet must only be used in conjunction with Risk Assessment No.34 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| TASK: PUMPING OUT TUNNEL SUMPS |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| SIGNIFICANT RISKS: (Detail all High & Medium Risks Here If This SSW is Not Attached to The Risk Assessment) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Is a ‘ Permit To Work’ required? (/ X) |  |  |  |  |  |  |  |  |  | NO |  |  | * |  |  |  | YES | * |  |  |  | Ref. Number: |  |  |  |  | * Documentation varies from job to job. Record data as appropriate, depending on the job. |  |  |  |  |  |  |  |  |  |
| Is a radiological controlled document required (/ X) |  |  |  |  |  |  |  |  |  | NO |  |  | * |  |  |  | YES | * |  |  |  | Ref. Number: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Is a designated work instruction required (/ X) |  |  |  |  |  |  |  |  |  | NO |  |  | * |  |  |  | YES | * |  |  |  | Ref. Number: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| P.P.E. Requirements: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ### Safety Helmet |  |  |  | Eye Protection |  |  |  |  | Face Masks |  |  |  |  |  |  | Safety Footwear |  |  |  |  |  |  |  |  | High Visibility Clothing |  |  |  |  |  |  | ### Gauntlets |  |  |  |  |
| Gloves |  | Hearing Defenders |  |  |  |  |  | Respirators |  |  |  |  |  | Chain Mail Gloves |  |  |  |  |  |  |  |  | All Weather clothing |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Other /PPE Details: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ADDITIONAL EQUIPMENT REQUIRED: Multigas meter, Sava-set (1 per individual in the tunnel) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| TASK MANAGEMENT: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| PROCEDURE: The WOC will provide general instructions for starting the job. If the system is likely to contain contaminated liquor arrange to collect a sample and have it tested. Obtain the tunnel access hatch key from SSCR. Make sure you take a multigas meter and sava-set with you even if all you are doing initially is taking a sample. After you have the results of the sample consult with Health Physics as to the safe disposition of the affluent being pumped. Assemble all the equipment needed, including the safety equipment. The members of the work party must initially contain a trained rigger/ slinger to insure the equipment is lowered into the tunnel safely. Set up the area prior to starting work. Prearrange collection vessels if the affluent must be taken to another sump or drain and disposed of. If the affluent is contaminated establish C2 areas where needed to assure radiological safety and plan for the containment of any possible leaks or spills. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Additional Training Required: Confined spaces, Weil’s disease, Sava-set use. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Additional Advice Available From: Senior Authorised Persons and Health Physics. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ‘SSW’ Prepared By: |  |  |  |  | Print |  |  |  |  |  |  |  |  |  |  |  |  |  | Sign |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Date ‘SSW’ Last Reviewed: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |