| #### Ref. CAT 52 | #### Task/Activity: Handling & Use of Balloon Gas (helium and air) |  |  |  | #### Assessment date: |
| --- | --- | --- | --- | --- | --- |
| #### Unit No: | #### Unit name/location: |  |  |  | #### Assessed by: |
| Hazard |  |  | Safety Risk | #### Safe System of Work / Control Measures |  |
| Manual Handling Hazards Falling Cylinders Changing/Connecting Cylinders Leakage Rapid release of gas resulting in uncontrolled movement of the cylinder |  |  | Back and muscle strain Crushing injuries Asphyxiation Impact injuries from being struck by the cylinder and/or colliding with fixed objects | - Balloon Gas cylinders only to be used and handled by employees trained in the content of this risk assessment - Balloon Gas cylinders to be stored, used and maintained in accordance with manufacturers / supplier’s MSDS / instructions. - Employees to be made aware of the safety information on the cylinder neck label and the warning on the side of the cylinder. - Balloon Gas cylinders only to be used with the correct regulators fitted. - Balloon Gas cylinders to be secured in an upright position using straps when in use. - Before use, the Balloon Gas valve should be momentarily opened a small amount and very slowly to blow out any grit or foreign matter. - Once connected to balloons, cylinder valves must be opened slowly. - If cylinders need to be checked to see if they are empty, the valve should be momentarily opened a small amount and very slowly to prevent any rapid release of Balloon Gas. - Storage areas to be indoors protected from extremes of heat and cold and well ventilated. - Damaged cylinders and regulators to be isolated and clearly labelled to prevent their use. - Do NOT allow anyone to breathe helium from cylinders or from filled balloons, although not poisonous it can result in asphyxiation. - Do NOT allow children or any unauthorised persons to handle cylinders or use balloon filling equipment. - Suitable cylinder trolleys or other equipment to be used to transport cylinders. Trolleys to be maintained in good condition and removed from use and labelled “Do Not Use” if defective. - Product Safety Data Sheet to be available. - Appropriate PPE, including steel toe capped safety shoes and stout gloves to be provided where handling cylinders forms a significant part of an employee’s duties. - Implement Safe System of Work/Control Measures in Trolleys, Sack Trucks etc, Ref. CRA 59 Risk Assessment, the COSHH Product Assessments and the manual handling safety precautions. |  |
| Who may be affected by the task/activity? (Tick all applicable boxes) |  |  |  |  |  |
| Compass employees/Agency staff |  |  |  |  |  |
| Customers / Client staff |  |  |  |  |  |
| Visitors / Members of the Public |  |  |  |  |  |
| Maintenance personnel |  |  |  |  |  |
| Delivery personnel |  |  |  |  |  |
| Other (specify below) |  |  |  |  |  |
| IMPORTANT - This risk assessment should be reviewed every 3 years, or whenever there is a significant change in the task or activity and following any accident or incident involving this task or activity. This risk assessment must be retained for a period of 6 years. |  |  |  |  |  |