| #### Ref. CAT 09 | #### Task/Activity: Use of Brat Pans |  |  |  | #### Assessment date: |
| --- | --- | --- | --- | --- | --- |
| #### Unit No: | #### Unit name/location: |  |  |  | #### Assessed by: |
| Hazard |  |  | Safety Risk | #### Safe System of Work / Control Measures |  |
| Electrical and/or gas hazards Contact with hot surfaces, hot food, liquid and steam Spillages of food and liquid when using the tilting mechanism Falling lid Manual handling hazards |  |  | Electric shock and burns Fire and explosion Burns and scalds Slips and falls Cuts and bruises and possible head injuries from being struck by the lid Back and muscle strains | - Brat pans only to be used and cleaned by trained employees. - Follow manufacturer's user instructions where these are available - Brat pans to be maintained by competent persons and in accordance with manufacturer's guidance where available - Brat pans not to be overfilled. - Tilting mechanism to be operated slowly to avoid spillages. - Avoid escaping steam when raising the lid. - Spillages to be cleaned up as they occur and wet floor warning signs used when appropriate. - Brat pans must not be used for deep fat frying. - When decanting food from the Brat pan, ensure that the receiving container is as close as possible to avoid spillages. - Containers not to be overfilled and whenever possible trolleys to be used to minimise manual handling risks. - Oven cloths/gloves to be used when handling hot equipment. - All operating switches to be conveniently accessible to the user. - Implement the Safe System of Work/Control Measures in Electrical Safety, Ref. MAN 05 or Gas Safety, Ref. COM 03, Trolleys, Sack Trucks etc, Ref. CAM 13 Risk Assessments and manual handling safety precautions. |  |
| Who may be affected by the task/activity? (Tick all applicable boxes) |  |  |  |  |  |
| Compass employees/Agency staff |  |  |  |  |  |
| Customers / Client staff |  |  |  |  |  |
| Visitors / Members of the Public |  |  |  |  |  |
| Maintenance personnel |  |  |  |  |  |
| Delivery personnel |  |  |  |  |  |
| Other (specify below) |  |  |  |  |  |
| IMPORTANT - This risk assessment should be reviewed every 3 years, or whenever there is a significant change in the task or activity and following any accident or incident involving this task or activity. This risk assessment must be retained for a period of 6 years. |  |  |  |  |  |