| #### Ref. CAT 26 | #### Task/Activity: Use of Free Standing Hot Water Urns |  |  |  | #### Assessment date: |
| --- | --- | --- | --- | --- | --- |
| #### Unit No: | #### Unit name/location: |  |  |  | #### Assessed by: |
| Hazard |  |  | Safety Risk | #### Safe System of Work / Control Measures |  |
| Contact with hot equipment and water Spillages Electrical hazards Manual handling hazards Free Standing Hot Water Urns used in this site are: |  |  | Burns and Scalds Slips and falls Electric shock Back and muscle strain | - Urns only to be used by trained employees. - Follow manufacturer's user instructions where these are available - Urns to be maintained by competent persons and in accordance with manufacturer's guidance where available, including regular descaling to avoid dripping taps. - Urns to be placed on a stable, level surface, away from the table edge and at a convenient height for the user. - Sufficient clearance between the tap and surface to allow the equipment being filled to be placed directly beneath the tap. - Suitable drip tray to be kept in position under the tap - Spillages to be cleaned up as they occur and wet floor warning signs used when appropriate. - Power sockets and leads to be positioned where they will not be affected by accidental spillages, water vapour and condensate. - Electrical leads to be positioned where they will not be accidentally pulled or create tripping hazards or strain placed on it or the connections, or be exposed to heat. - The urn to be filled using an appropriate method, e.g. jug, pan, bucket, hose, to avoid manual handling injuries and spillages. - Urns not to be allowed to boil dry - Empty the contents of the urn via the dispense tap and allow it to cool before moving. - Care to be taken when removing the lid to avoid contact with steam and drops of hot water from the underside of the lid - Implement the Safe System of Work/Control Measures in the Electrical Safety Risk Assessment, Ref. MAN 05, including an annual PAT test and the manual handling safety precautions |  |
| Who may be affected by the task/activity? (Tick all applicable boxes) |  |  |  |  |  |
| Compass employees/Agency staff |  |  |  |  |  |
| Customers / Client staff |  |  |  |  |  |
| Visitors / Members of the Public |  |  |  |  |  |
| Maintenance personnel |  |  |  |  |  |
| Delivery personnel |  |  |  |  |  |
| Other (specify below) |  |  |  |  |  |
| IMPORTANT - This risk assessment should be reviewed every 3 years, or whenever there is a significant change in the task or activity and following any accident or incident involving this task or activity. This risk assessment must be retained for a period of 6 years. |  |  |  |  |  |