| #### Ref. CAT 51 | #### Task/Activity: Working in Coldrooms, Walk-in Chillers and Freezers |  |  |  | #### Assessment date: |
| --- | --- | --- | --- | --- | --- |
| #### Unit No: | #### Unit name/location: |  |  |  | #### Assessed by: |
| Hazard |  |  | Safety Risk | #### Safe System of Work / Control Measures |  |
| Low temperatures Ice and melting ice Goods stored on floor surface Poor lighting Falling objects Trapping in confined space Manual handling |  |  | Thermal discomfort from prolonged working at low temperatures Trips, slips and falls Impact injuries Asphyxiation Back and muscle strain | - Follow manufacturer's user instructions where these are available - Coldrooms, walk-in chillers and freezers to be maintained by competent persons and in accordance with manufacturer's guidance where available - Employees to work only for short periods of time in coldrooms whenever practicable. - Where prolonged working at low temperatures is necessary, employees to be provided with suitable thermal footwear and clothing and given frequent breaks in a warm area where hot drinks can be consumed. - Where applicable, coldrooms, walk-in chillers and freezers to be de-frosted at appropriate intervals to prevent the build up of ice forming on the condenser unit, the floor surface and other parts. - Where heavy packs of frozen goods become fused together with forming ice, these should be separated before attempting to lift and carry. - Water and ice to be cleaned up to reduce slipping hazards. - Coldrooms, walk-in chillers and freezer doors to be fitted with a door release device accessible from inside and employees trained in its use in the event of the door being closed when they are inside. - Where practicable, the door release device should be marked by a luminous sign or emergency lighting so that it is clearly visible to anyone trapped inside. The door release device must be tested weekly to ensure its effective operation. - Where practicable, an audible alarm should be fitted so that it can be sounded from inside the coldroom in the event of the door release device failing to operate. - A physical check must be made to confirm there is no one inside when, for reasons of security, cold rooms are locked or padlocked at the end of the working day. - No person who is working alone in the unit should work inside a coldroom with the door closed. - Goods to be stored off the floor area used as the main walkway into and out of the coldroom. - Racking to be of adequate strength and goods to be stored securely so that there is no risk of items falling. - Adequate lighting to be provided inside ccoldrooms, walk-in chillers and freezers and defective bulbs or fluorescent tubes to be replaced without undue delay. - Implement the manual handling safety precautions |  |
| Who may be affected by the task/activity? (Tick all applicable boxes) |  |  |  |  |  |
| Compass employees/Agency staff |  |  |  |  |  |
| Customers / Client staff |  |  |  |  |  |
| Visitors / Members of the Public |  |  |  |  |  |
| Maintenance personnel |  |  |  |  |  |
| Delivery personnel |  |  |  |  |  |
| Other (specify below) |  |  |  |  |  |
| IMPORTANT - This risk assessment should be reviewed every 3 years, or whenever there is a significant change in the task or activity and following any accident or incident involving this task or activity. This risk assessment must be retained for a period of 6 years. |  |  |  |  |  |