| #### Ref. COM 15 | #### Task/Activity: Use of Hand-Pallet Trucks |  |  |  | #### Assessment date: |
| --- | --- | --- | --- | --- | --- |
| #### Unit No: | #### Unit name/location: |  |  |  | #### Assessed by: |
| Hazard |  |  | Safety Risk | #### Safe System of Work / Control Measures |  |
| Manual handling hazards Falling loads Striking people, stationary objects and structures Overloaded and unevenly loaded trucks Uneven and sloping routes |  |  | Back, muscle and upper limb strains Crushing injuries to feet Impact injuries Cuts and bruises Slips, trips and falls Possible fractures to bones | - Pallet trucks to be used only by trained employees. - Follow manufacturer's user instructions where these are available - Pallet Trucks to be maintained by competent persons and in accordance with manufacturer's guidance. - Wheels and castors maintained in a clean condition to ensure their free running. - Wheels to be checked to ensure they are free running before use. - Floors and walkways to be maintained to keep them clear of obstructions. - Pallet truck forks to be fully engaged under the pallet before lifting - Pallet trucks must not be overloaded. Employees must be trained to be able to test or assess the weight of a pallet before moving it. - Loaded pallet trucks to be moved by pulling rather than by pushing, except when manoeuvring in tight situations. - Avoid using pallet trucks on slopes or uneven surfaces, where possible. Obtain assistance if necessary. - Employees must ensure that the load is stacked securely. Do not continue with moving if the load becomes unstable. Get assistance. - Employees must ensure that surrounding area is clear of risk of trapping people’s feet before lowering. - When not in use, pallet trucks must be safely stored away from walkways, fire escape routes and traffic routes. - Spillages to be cleaned up as they occur and wet floor warning signs used when appropriate. - Implement manual handling safety precautions. - Employees operating pallet trucks must wear safety footwear with reinforced toe caps. - Employees must not ride on the load or the forks. |  |
| Who may be affected by the task/activity? (Tick all applicable boxes) |  |  |  |  |  |
| Compass employees/Agency staff |  |  |  |  |  |
| Customers / Client staff |  |  |  |  |  |
| Visitors / Members of the Public |  |  |  |  |  |
| Maintenance personnel |  |  |  |  |  |
| Delivery personnel |  |  |  |  |  |
| Other (specify below) |  |  |  |  |  |
| IMPORTANT - This risk assessment should be reviewed every 3 years, or whenever there is a significant change in the task or activity and following any accident or incident involving this task or activity. This risk assessment must be retained for a period of 6 years. |  |  |  |  |  |