| #### Ref. CAT 24 | #### Task/Activity: Use and Cleaning of Food Mixing Machines (Planetary) |  |  |  | #### Assessment date: |
| --- | --- | --- | --- | --- | --- |
| #### Unit No: | #### Unit name/location: |  |  |  | #### Assessed by: |
| Hazard |  |  | Safety Risk | #### Safe System of Work / Control Measures |  |
| Contact with moving machine parts Striking with ejected food ingredients Instability of mixing machine Electrical hazards Manual handling Food Mixers in use in this site are: |  |  | Entanglement Cuts and lacerations Crush injuries Amputation Impact injuries Trapping or crushing under fallen machine Electric shock Back and muscle strain | - Food mixers and food processing attachments, e.g. graters, slicers, mincers etc, only to be used by trained employees. - Food mixers only to be dismantled and cleaned by trained employees of 18 years of age or over. - Follow manufacturer's user instructions where these are available - Food mixers to be maintained by competent persons and in accordance with manufacturer's guidance where available - Food mixers to be fitted with all necessary guards and safety interlocks, which must be in good working order. - Employees must not reach into the mixing bowl or attachments whilst the machine is operating and must ensure that the motor has stopped and parts are not moving before placing hands or utensils into the bowl or attachment and before opening interlocked guards where fitted. - Older models that do not have interlocked guards must be used with a bowl extension ring or collar fitted. - Push tools to be used to prevent hands coming into contact with moving blades or attachments. - Food mixers to be securely sited to ensure stability, with adequate workspace. - A sign to be displayed adjacent to the machine and in view of the operator stating “Unauthorised persons not to use this machine”. - All operating and isolating switches to be conveniently accessible to the user. - Protective clothing and cloths etc to be kept away from moving machine parts so that they cannot become entangled and drawn into the machine. - Castor-framed trolleys to be used to move heavy and/or large mixing bowls. - Implement the Safe System of Work/Control Measures in Electrical Safety Risk Assessment, Ref. MAN 05, including an annual PAT test and the manual handling safety precautions |  |
| Who may be affected by the task/activity? (Tick all applicable boxes) |  |  |  |  |  |
| Compass employees/Agency staff |  |  |  |  |  |
| Customers / Client staff |  |  |  |  |  |
| Visitors / Members of the Public |  |  |  |  |  |
| Maintenance personnel |  |  |  |  |  |
| Delivery personnel |  |  |  |  |  |
| Other (specify below) |  |  |  |  |  |
| IMPORTANT - This risk assessment should be reviewed every 3 years, or whenever there is a significant change in the task or activity and following any accident or incident involving this task or activity. This risk assessment must be retained for a period of 6 years. |  |  |  |  |  |