| #### Ref. COM 11 | #### Task/Activity: Use of Step Stools (Page 1 of 2) |  |  |  | #### Assessment date: |
| --- | --- | --- | --- | --- | --- |
| #### Unit No: | #### Unit name/location: |  |  |  | #### Assessed by: |
| Hazard |  |  | Safety Risk | #### Safe System of Work / Control Measures |  |
| Falling from height Falling objects Manual handling |  |  | Bruising Bone fractures Concussion Flesh wounds Slips and Falls Back and muscle strain | A step stool may only be used if: - There is no suitable safer alternative means to access the task - The work task is of short duration - It is used for light work where a maximum load of 10kg is carried - Employees have been trained in the correct selection, positioning and safe use, including the carrying out of safety checks before use - A visual check has been made by the user to ensure that the step stool is in a satisfactory condition Step stool selection: - Must be of a suitable size for the task. - Must be suitable for the surface conditions on which it will be used, e.g. suitable for use on smooth surfaces or floor tiles - Must have a sound slip resistant grip to the step surface Positioning of step stool: - Position on a firm level surface, free from loose materials, spillages and obstructions - Position where it is not at risk of being pushed or knocked by other hazards, e.g. opening doors. If necessary, secure doors or have a person stand guard during the task. If necessary erect suitably located warning signs/barriers as appropriate to warn or prevent unauthorised access when in use - Position the step stool to face the work |  |
| Who may be affected by the task/activity? (Tick all applicable boxes) |  |  |  |  |  |
| Compass employees/Agency staff |  |  |  |  |  |
| Customers / Client staff |  |  |  |  |  |
| Visitors / Members of the Public |  |  |  |  |  |
| Maintenance personnel |  |  |  |  |  |
| Delivery personnel |  |  |  |  |  |
| Other (specify below) |  |  |  |  |  |
| IMPORTANT - This risk assessment should be reviewed every 3 years, or whenever there is a significant change in the task or activity and following any accident or incident involving this task or activity. This risk assessment must be retained for a period of 6 years. |  |  |  |  |  |

| Ref. COM 11 | Task/Activity: Use of Step Stools (Page 2 of 2) |  |  |
| --- | --- | --- | --- |
| Hazard |  | Safety Risk | #### Safe System Of Work / Control Measures |
| Falling from height Falling objects Manual handling |  | Bruising, Bone fractures, Concussion, Flesh wounds. Slips and Falls Back and muscle strain | Safe use of step stools - The user must check that step stool is in good condition before use - If it is found to be defective it must not be used and the fault must be immediately reported to the manager or supervisor - Never overreach when working on a step stool - Do not attempt to lift an object that is beyond personal capability - The user must face the step stool at all times when standing on and off it - Stand on the step with both feet firmly in contact with the step surface - Always wear suitable clean footwear with a good sole grip and a low heel - After use, return it to a secure place of storage - Do not leave it in the work area or any other place where it may present a tripping hazard |