| #### Ref. COM 10 | #### Task/Activity: Use of Steam Cleaners |  |  |  | #### Assessment date: |
| --- | --- | --- | --- | --- | --- |
| #### Unit No: | #### Unit name/location: |  |  |  | #### Assessed by: |
| Hazard |  |  | Safety Risk | #### Safe System of Work / Control Measures |  |
| Electrical hazards Contact with hot surfaces and steam Spillages of water/ wet surfaces Trailing cables Pressure vessels Manual handling Cleaning products Steam Cleaners used in this site are: |  |  | Electric shock Burns and scalds Slips, trips and falls Explosion Back and muscle strain Irritation to skin and eyes | - Steam cleaners only to be used and cleaned by trained employees. - Manufacturer's user instructions to be followed where these are available - Steam cleaners to be maintained by competent persons and in accordance with manufacturer's guidance where available. Pressurised steam cleaners to be inspected in accordance with a written scheme of examination by a competent person at appropriate intervals. - Suitable Personal Protective Equipment to be worn as per manufacturer’s user instructions. Use at all times, if damaged or mislaid, report it to your manager. - Steam cleaners always to be used in conjunction with a residual current device (RCD). - Reduce the length of trailing cables and minimise the use of extension leads by using the nearest electrical socket outlet. Cables to be kept away from the area being steam cleaned and behind the user. - Ensure no strain is placed on the electrical cable and connections. - Do not point the steam nozzle at any part of the body or towards any other person. - Do not direct the steam nozzle close to electrical equipment or sockets. - Wherever practicable, steam cleaners to be used when the area of floor being cleaned will be subject to the least possible pedestrian movement. Erect suitably located warning signs/barriers as appropriate to warn or prevent unauthorised access when cleaning in progress. Additional warning signs will be required when cleaning staircases. Ensure the floor is completely dry before removing signage. - Do not leave the machine unattended in a public area. - Disconnect from the mains electrical supply and ensure the cleaner is cool before assembling, cleaning or maintaining the steam cleaner. - Implement the Safe System of Work/Control Measures in the Electrical Safety Risk Assessment, Ref. MAN 05, COSHH Product Assessments and the manual handling safety precautions |  |
| Who may be affected by the task/activity? (Tick all applicable boxes) |  |  |  |  |  |
| Compass employees/Agency staff |  |  |  |  |  |
| Customers / Client staff |  |  |  |  |  |
| Visitors / Members of the Public |  |  |  |  |  |
| Maintenance personnel |  |  |  |  |  |
| Delivery personnel |  |  |  |  |  |
| Other (specify below) |  |  |  |  |  |
| IMPORTANT - This risk assessment should be reviewed every 3 years, or whenever there is a significant change in the task or activity and following any accident or incident involving this task or activity. This risk assessment must be retained for a period of 6 years. |  |  |  |  |  |