| #### Ref. CAT 04 | #### Task/Activity: Barbecues (LPG and Charcoal) |  |  |  | #### Assessment date: |
| --- | --- | --- | --- | --- | --- |
| #### Unit No: | #### Unit name/location: |  |  |  | #### Assessed by: |
| Hazard |  |  | Safety Risk | #### Safe System of Work / Control Measures |  |
| Hot surfaces, utensils and equipment Hot foods and liquids Food spillages Wet, slippery surfaces Obstructions/Tripping Hazards Liquefied Petroleum Gas (LPG) Manual Handling |  |  | Burns and scalds Slips, trips and falls Explosion & fire Back and muscle strain | - Gas barbecues to be maintained in good working order by competent persons. - Where possible, supplier to set up and dismantle gas barbecues. - Only trained employees to operate the barbecue - Barbecue to be sited away from buildings and adjacent windows kept closed. - Site of barbecue to be as dry as reasonably practicable and clear of obstructions that may cause tripping accidents. - Barbecues and LPG cylinders to be sited on a firm, level surface where unable to be accidentally interfered with and secured to prevent them from falling over. - Check for gas leaks to be carried out immediately prior to use by listening for a hissing sound, smelling for gas and by applying a 50/50 solution of washing up liquid and water onto hose connections and looking to see if “bubbles” form. - Customers to be kept well clear of hot surfaces by using barriers and/or suitable signage if necessary. Congestion around the barbecue minimised by organisation and supervision of customer traffic routes. Clear route from kitchen to barbecue to be maintained. - Cooking utensils to be stored so that they cannot fall or cause a tripping hazard.. - Long handled utensils to be used to maintain hands and arms a safe distance from hot food and equipment when cooking. - Chefs to wear jackets/tunics with long sleeves. - PPE to be used e.g. oven gloves/cloths that are dry and in good condition - Barbecues must not be left unattended during cooking. - Sufficient tables or work surfaces to be provided for food and equipment not being cooked. - A sufficient number of suitable bins to be provided for waste. - Food spillages to be cleaned up as they occur and warning signs used when appropriate. - First aid facilities to be readily available, including clean cold water for the treatment of burns and scalds. - Fire fighting equipment to be readily available. - Implement the Safe System of Work/Control Measures in the Cooking Risk Assessment, Ref. CAT 15, Storage and Use of LPG in Cylinders Risk Assessment, Ref. COM 12 and manual handling safety precautions. |  |
| Who may be affected by the task/activity? (Tick all applicable boxes) |  |  |  |  |  |
| Compass employees/Agency staff |  |  |  |  |  |
| Customers / Client staff |  |  |  |  |  |
| Visitors / Members of the Public |  |  |  |  |  |
| Maintenance personnel |  |  |  |  |  |
| Delivery personnel |  |  |  |  |  |
| Other (specify below) |  |  |  |  |  |
| IMPORTANT - This risk assessment should be reviewed every 3 years, or whenever there is a significant change in the task or activity and following any accident or incident involving this task or activity. This risk assessment must be retained for a period of 6 years. |  |  |  |  |  |