| #### Ref. COM 14 | #### Task/Activity: Use of Tugs |  |  |  | #### Assessment date: |
| --- | --- | --- | --- | --- | --- |
| #### Unit No: | #### Unit name/location: |  |  |  | #### Assessed by: |
| Hazard |  |  | Safety Risk | #### Safe System of Work / Control Measures |  |
| Overloaded or unbalanced loads Driving too fast and/or cornering too sharply Defective or incorrectly fitted couplings Battery acid leakage in the event of an accident Manual handling |  |  | Tug overturning and collisions with other vehicles, pedestrians and fixed objects, resulting in: - Death and serious injuries to driver (and occupants) of vehicle - Death and serious injuries to drivers of other vehicles and pedestrians - Damage to vehicle and property. Struck by falling/moving object Acid burns Back and muscle strains | - Tugs only to be operated by trained employees aged 18 years or over. - Tugs to be maintained by a competent trained person, in accordance with manufacturer’s guidance, where available - The maximum number of containers that may be towed at a time should be established and communicated to all drivers, taking into account site layout and conditions. Wherever practicable, designated safe routes around the site should be established, documented and communicated to all drivers. - Drivers to notify their line manager of any circumstances including medical conditions or medication being taken that may affect their ability to drive. - Drivers must not drive tugs whilst under the influence of alcohol or drugs. - Drivers to carry out basic safety checks before using vehicles (brake operations, horn, mirrors, light operation, indicators and reversing alarms where fitted). - Drivers to complete the Weekly Vehicle Checklist. - Drivers to check security of couplings between tug and trailer and between trailers before moving off. - All defects that may affect health and safety to be reported and remedied before further use. Records of all repairs and servicing to be maintained. - Drivers to obey normal road traffic regulations, Highway Code, site-specific regulations as appropriate when driving the vehicle. - Sufficient time must be allowed for the journey to be made without the need for undue speed. - When tugs are left unattended they must be parked in a safe place with the parking brake on and the ignition keys removed. - Implement the Safe System of Work/Control Measures in the Charging Lead-Acid Batteries, Ref. COM 02, Risk Assessment and the safety precautions in the Workplace Transport Assessment and the manual handling safety precautions |  |
| Who may be affected by the task/activity? (Tick all applicable boxes) |  |  |  |  |  |
| Compass employees/Agency staff |  |  |  |  |  |
| Customers / Client staff |  |  |  |  |  |
| Visitors / Members of the Public |  |  |  |  |  |
| Maintenance personnel |  |  |  |  |  |
| Delivery personnel |  |  |  |  |  |
| Other (specify below) |  |  |  |  |  |
| IMPORTANT - This risk assessment should be reviewed every 3 years, or whenever there is a significant change in the task or activity and following any accident or incident involving this task or activity. This risk assessment must be retained for a period of 6 years. |  |  |  |  |  |