| #### Ref. COM 02 | #### Task/Activity: Charging Lead-Acid Batteries |  |  |  | #### Assessment date: |
| --- | --- | --- | --- | --- | --- |
| #### Unit No: | #### Unit name/location: |  |  |  | #### Assessed by: |
| Hazard |  |  | Safety Risk | #### Safe System of Work / Control Measures |  |
| Charging of lead-acid batteries used on tugs and other battery powered equipment. Contact with battery acid Electrical hazards |  |  | Fire/explosion due to ignition of hydrogen produced during the charging process. Burns to skin and eyes from battery acid. Electric shock and burns | - Battery charging only to be carried out by trained employees. - Follow manufacturer's user instructions where these are available - Chargers to be maintained by competent persons and in accordance with manufacturer's guidance, where available - Employees must not wear jewellery, metal rings or watches when handling batteries. - Separate room or area to be designated for charging batteries. - Appropriate PPE to be available and worn when topping up the battery cells e.g. goggles, gauntlets, apron, disposable respirator mask rated to FFP2 standard. - Emergency eyewash station to be located in the battery charging area. - All non-essential people excluded from the battery charging area. - Sufficient high level ventilation provided above the batteries. - Electrical appliances and any other potential sources of ignition situated distant from the charging area and/or sited below the level of the batteries, but not in a position where spillage could fall onto the electrical appliance. - "No Smoking" and "No Naked Lights" signage to be displayed inside the battery charging area and adjacent to it - To avoid sparks, the charger should be switched off before connection and disconnection from the battery. - Tools must not touch both battery terminals at the same time. - Implement the Safe System of Work/Control Measures in Electrical Safety Risk Assessment, Ref MAN 05, including an annual PAT test where appropriate. |  |
| Who may be affected by the task/activity? (Tick all applicable boxes) |  |  |  |  |  |
| Compass employees/Agency staff |  |  |  |  |  |
| Customers / Client staff |  |  |  |  |  |
| Visitors / Members of the Public |  |  |  |  |  |
| Maintenance personnel |  |  |  |  |  |
| Delivery personnel |  |  |  |  |  |
| Other (specify below) |  |  |  |  |  |
| IMPORTANT - This risk assessment should be reviewed every 3 years, or whenever there is a significant change in the task or activity and following any accident or incident involving this task or activity. This risk assessment must be retained for a period of 6 years. |  |  |  |  |  |