| #### Ref. COM 20 | #### Task/Activity: Use of Stepladders (Page 1 of 4) |  |  |  | #### Assessment date: |
| --- | --- | --- | --- | --- | --- |
| #### Unit No: | #### Unit name/location: |  |  |  | #### Assessed by: |
| Hazard |  |  | Safety Risk | #### Safe System of Work / Control Measures |  |
| Falling from height Falling objects Manual handling |  |  | Death, bruising, bone fractures, concussion, flesh wounds. Back and muscle strain | ###### Suitable activities for using stepladders Wherever possible, working at height and working from stepladders should be avoided, for example, by storing items at lower levels, taking items down to ground level to repair/maintain etc. Where working at height cannot be avoided precautions must be taken to prevent a person from falling. A stepladder may only be used if: - There is no suitable safer alternative means to access the task, e.g. fixed scaffold or tower scaffold, scissor lift, cherry picker etc; and - The work task is of short duration and the stepladder is in one position for no more than 30 minutes. - It is used for light work where a maximum load of 10kg is carried on the stepladder. - Employees using stepladders are physically fit and have been trained in their correct selection, positioning and safe use, including the carrying out of safety checks before use. - The stepladder details are entered onto the Stepladder Register and a Management Safety Check has been carried out within the last 6 months. (See page 4 of this risk assessment) - A visual check has been made by the user to ensure that the stepladder is in a satisfactory condition. Stepladder selection - Stepladders must be of a suitable size for the task. - Stepladders must be suitable for the surface conditions on which they will be used, e.g. suitable for use on smooth surfaces or floor tiles, referring to the manufacturer’s user instructions. - If the total load on a stepladder is greater than 95Kg (including the person), a stepladder rated as Class 1 (Industrial) or to EN: 131 must be used. CONTINUED ON PAGE 2 |  |
| Who may be affected by the task/activity? (Tick all applicable boxes) |  |  |  |  |  |
| Compass employees/Agency staff |  |  |  |  |  |
| Customers / Client staff |  |  |  |  |  |
| Visitors / Members of the Public |  |  |  |  |  |
| Maintenance personnel |  |  |  |  |  |
| Delivery personnel |  |  |  |  |  |
| Other (specify below) |  |  |  |  |  |
| IMPORTANT - This risk assessment should be reviewed every 3 years, or whenever there is a significant change in the task or activity and following any accident or incident involving this task or activity. This risk assessment must be retained for a period of 6 years. |  |  |  |  |  |

| Ref. COM 20 | Task/Activity: Use of Stepladders (Page 2 of 4) |  |  |
| --- | --- | --- | --- |
| Hazard |  | Safety Risk | #### Safe System Of Work / Control Measures |
|  |  |  | Positioning of stepladders - Stepladders must only be used on firm level ground, free from loose materials, spillages and obstructions. - Stepladders must be positioned where they are not at risk of being pushed over by other hazards, e.g. opening doors or windows. If necessary, secure doors and windows or have a person stand guard during the task. - Erect suitably located warning signs/barriers as appropriate to warn or prevent unauthorised access when in use. - Ensure that the platform, where provided, is locked into position on the rear cross tube. - Ensure that the stepladder is fully opened with cross-members or ropes fully extended. - Stepladders must not be placed on moveable objects such as pallets, bricks, lift trucks, tower scaffolds, vans, or mobile elevating work platforms. - Wherever practicable, position the stepladder to face the work. - Where facing the work is not practicable and where the work on stepladders imposes a side loading, e.g. drilling, either securely tie the stepladder to prevent it tipping over or ensure that another person holds the stepladder securely. - Stepladders must not be used in strong winds or in adverse weather conditions where there is an increased risk of slipping on the rungs, e.g. heavy rain, ice or snow. CONTINUED ON PAGE 3 |

| Ref. COM 20 | Task/Activity: Use of Stepladders (Page 3 of 4) |  |  |
| --- | --- | --- | --- |
| Hazard |  | Safety Risk | #### Safe System Of Work / Control Measures |
|  |  |  | Safe use of stepladders - The user must check that stepladders are in good repair before use (rubber feet in place and not damaged; no damage to steps, platform or cross tubes; no damage to bracings). - If a stepladder is found to be defective it must not be used and the fault must be immediately reported to the manager or supervisor. - When using a stepladder the user must avoid standing on the top three steps, unless a suitable handrail is available or there is a safe working platform with a handrail. - Do not stand on the top handrail or cross tubes. - Ask for assistance if required, to hold the stepladder. - Do not attempt to lift an object that is beyond personal capability whilst standing on a stepladder. - The user must face the stepladder at all times when going up or down. - Tool belts or carriers should be used to reduce the risk of dropping tools. - Never overreach when working on a stepladder. - Never leave stepladders unattended where there is a risk of unauthorised persons using them. - Always wear suitable clean footwear when using stepladders. - Do not “jump” the stepladder to move it, always reposition it from the ground. - After use, return the stepladder to a secure place of storage, protected from the effects of the weather. - Implement the Safe System of Work/Control Measures in Lone Working Risk Assessment, Ref. CRA 38 and the manual handling safety precautions For further guidance refer to the HSE Guidance Section of the Catering Health and Safety Manual in Compass Directory on the Intranet: - HSE Indg documents 401, 402, 403 & 405 relating to working at heights and use of ladders and stepladders Stepladder Register and Safety Check is on Page 4 |

# Ref. COM 20						STEPLADDER REGISTER AND SAFETY CHECK

| Site Name: |  |  |  |  |  |  |  | Unit No: |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| # STEPLADDER DETAILS |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Stepladder Number: |  |  |  | Stepladder Make/Type: |  |  |  | Duty/Weight/Class Rating: |  |  |  | Date first put into use: |  |  |  |
| # 6 MONTHLY MANAGEMENT SAFETY CHECK |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Date | Condition of Stiles and Steps |  | Missing or Loose Rivets |  |  | Condition of Bracings/Cross-members/Ropes |  |  | Condition of Feet |  | Signs of Corrosion |  |  | Correctly Stored |  |
|  | Satisfactory |  | Satisfactory |  |  | Satisfactory |  |  | Satisfactory |  | Satisfactory |  |  | Satisfactory |  |
|  | Yes | No | Yes |  | No | Yes | No |  | Yes | No | Yes |  | No | Yes | No |
| Where stepladders are in an unsatisfactory condition record corrective active taken: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

This Stepladder Register and Safety Check must be retained for a period of 3 years