| #### Ref. CAT 47 | #### Task/Activity: Use of Bread Slicing Machines |  |  |  | #### Assessment date: |
| --- | --- | --- | --- | --- | --- |
| #### Unit No: | #### Unit name/location: |  |  |  | #### Assessed by: |
| Hazard |  |  | Safety Risk | #### Safe System of Work / Control Measures |  |
| Exposed cutting blade Incorrect use of bread slicing machine Entanglement Electrical hazards Manual handling |  |  | Cuts and lacerations Amputation Bruising Electric shock Back and muscle strain | - Bread slicing machines only to be used by trained employees. - Bread slicing machines only to be dismantled and cleaned by trained employees of 18 years of age or over. - Follow manufacturer's user instructions where these are available - Bread slicing machines to be maintained by competent persons and in accordance with manufacturer's guidance where available - Bread slicing machines to be securely sited to ensure stability, with adequate workspace and lighting. - All operating and isolating switches to be conveniently accessible to the user. - Slicing machines to be fitted with all necessary guards and safety interlocks, which must be in good working order. - Where bread slicing machines operate by closing a hinged cover to start the machine the cover must be fitted with an interlock device that will only allow the machine to start when the cover is fully closed. - The bread slicing machine must be sited where the interlocked hinged cover cannot be accidentally closed by knocking into it or be pushing the machine against a fixed object or wall. - Where bread slicing machines are sited on a wheeled trolley/stand the wheels must be the locked to prevent the machine from moving when in use. - The machine must be isolated from the electricity supply before releasing the locking wheels to move the machine and trolley/stand for any purpose. - The machine must be isolated from the electricity supply before attempting to clear any blockage or obstruction from inside the machine. - Employees to ensure that clothing cannot become entangled in bread slicing machines. - Bread slicing machines must not be used for any purpose other than for slicing bread. - Size and shape of bread to be sliced must be suitable for the machine. - Implement the Safe System of Work/Control Measures in Electrical Safety Risk Assessment, Ref. MAN 05, including an annual PAT test. - Implement manual handling safety precautions where bread slicing machines need to be moved to and from their place of use. |  |
| Who may be affected by the task/activity? (Tick all applicable boxes) |  |  |  |  |  |
| Compass employees/Agency staff |  |  |  |  |  |
| Customers / Client staff |  |  |  |  |  |
| Visitors / Members of the Public |  |  |  |  |  |
| Maintenance personnel |  |  |  |  |  |
| Delivery personnel |  |  |  |  |  |
| Other (specify below) |  |  |  |  |  |
| IMPORTANT - This risk assessment should be reviewed every 3 years, or whenever there is a significant change in the task or activity and following any accident or incident involving this task or activity. This risk assessment must be retained for a period of 6 years. |  |  |  |  |  |