| #### Ref. COM 18 | #### Task/Activity: Use of Waste Compactors |  |  |  | #### Assessment date: |
| --- | --- | --- | --- | --- | --- |
| #### Unit No: | #### Unit name/location: |  |  |  | #### Assessed by: |
| Hazard |  |  | Safety Risk | #### Safe System of Work / Control Measures |  |
| Trapping Entanglement Electrical Hazards Manual handling Waste Compactors used in this site are: |  |  | Cuts Crushing injuries Amputation of hands/fingers Electric shocks Back and muscle strain | - Equipment only to be used and cleaned by trained persons who are 18 years of age or over. - Compactors to be used, cleaned and maintained in accordance with manufacturer's guidance where available - Equipment to be fitted with all necessary guards and safety interlocks, which must be in good working order - All operating switches to be conveniently accessible to the user - Only waste items for which the compactor is designed to compress are to be placed into the machine, e.g. no aerosol cans or other pressurised containers. - A sign to be displayed adjacent to the machine and in view of the operator stating “Unauthorised persons not to use this machine”. - Waste compactors to be isolated from the power supply before cleaning and before attempting to remove any obstruction. - The area around the waste compactor to be maintained in a clean and tidy condition. - Implement the Safe System of Work/Control Measures in Disposal of General Waste, Ref. COM 06 and Electrical Safety, Ref. MAN 05 Risk Assessments, including an annual PAT test, if appropriate. - Implement the manual handling safety precautions. |  |
| Who may be affected by the task/activity? (Tick all applicable boxes) |  |  |  |  |  |
| Compass employees/Agency staff |  |  |  |  |  |
| Customers / Client staff |  |  |  |  |  |
| Visitors / Members of the Public |  |  |  |  |  |
| Maintenance personnel |  |  |  |  |  |
| Delivery personnel |  |  |  |  |  |
| Other (specify below) |  |  |  |  |  |
| IMPORTANT - This risk assessment should be reviewed every 3 years, or whenever there is a significant change in the task or activity and following any accident or incident involving this task or activity. This risk assessment must be retained for a period of 6 years. |  |  |  |  |  |