| # Ref. COM 03 | Task/Activity: Gas Safety |  |  |  | Assessment date: |
| --- | --- | --- | --- | --- | --- |
| Unit No: | Unit name/location: |  |  |  | Assessed by: |
| Hazard |  |  | Safety Risk | Safe System of Work / Control Measures |  |
| Hot flame and equipment Carbon monoxide Flash back Gas leaks Gas Appliances used in this site are: |  |  | Burns Asphyxiation, carbon monoxide poisoning Death Fire and explosion | - Gas installations, connections and equipment to be installed, inspected, maintained and serviced by competent persons who are GAS SAFE Registered. Routine inspection and servicing to be in accordance with manufacturers recommendations. This should be carried out at least annually. - Each gas appliance should have a conveniently accessible isolating valve that will permit it to be turned off whilst not affecting other equipment. - Where it is reasonably practicable to do so, gas appliances should be fitted with flame failure devices. These will cut off the supply of gas to the burner if the flame is extinguished thereby preventing asphyxiation or explosion. - Workplaces where gas equipment is used must be adequately ventilated to ensure correct combustion and removal of the waste products from combustion. - Gas equipment only to be used by trained employees in accordance with manufacturer’s instructions where provided. This training to include the correct selection of equipment for the tasks undertaken, lighting, cleaning and emergency procedures. - Where gas equipment has to be lit manually, a taper must be used. The taper must be lit and presented to the burner at arms length immediately the gas is turned on. If the appliance fails to light, ventilate the equipment for at least 3 minutes before attempting to light it again so that any residual gas has had time to disperse. - All employees must know where the main gas-isolating valve is situated so that the gas can be turned off in an emergency. If a gas leak is suspected, all appliances should be turned off, light switches or electrical equipment must not be switched on or off, if possible the room should be ventilated to the outside air, the room vacated and a competent gas engineer called to investigate. - Where gas equipment faults pose a risk of injury, they must be turned off at the isolating valve and labelled and not used again until repaired by a competent person. - Checks to be made to ensure that gas appliances are turned off where appropriate at the end of the day. |  |
| Who may be affected by the task/activity? (Tick all applicable boxes) |  |  |  |  |  |
| Compass employees/Agency staff |  |  |  |  |  |
| Customers / Client staff |  |  |  |  |  |
| Visitors / Members of the Public |  |  |  |  |  |
| Maintenance personnel |  |  |  |  |  |
| Delivery personnel |  |  |  |  |  |
| Other (specify below) |  |  |  |  |  |
| IMPORTANT - This risk assessment should be reviewed every 3 years, or whenever there is a significant change in the task or activity and following any accident or incident involving this task or activity. This risk assessment must be retained for a period of 6 years. |  |  |  |  |  |