| # Ref. POR 04 | Task/Activity: Medical Gases (Page 1 of 2) |  |  |  | Assessment date: |
| --- | --- | --- | --- | --- | --- |
| Unit No: | Unit name/location: |  |  |  | Assessed by: |
| Hazard |  |  | Safety Risk | Safe System of Work / Control Measures |  |
| Manual Handling Falling cylinders Leaking cylinders Medical gases handled in this site are: |  |  | Back and muscle strain Crushing or impact injuries Leakage of gas, giving rise to: - Asphyxiation - Poisoning - Explosion - Fire - Cold Burns | - Medical gases only to be handled by trained staff - Safety shoes with steel toecaps to be provided and worn by all staff that handle medical gas cylinders as a significant part of their work duties - Follow supplier/manufacturer instructions where these are available - Stored cylinders to be harnessed in an upright position or on suitable cylinder racks or laid flat and chocked to prevent movement within designated storage locations - Full and empty cylinders to be stored separately and the segregation clearly identifiable. - Designated storage areas to be provided with good ventilation at both high and low level. - Where appropriate, an audible or visual alarm together with appropriate warning signage should be fitted in storage area to warn of low oxygen levels below 20% in the event of gases leaking into the atmosphere. - Storage locations to be kept locked and warning signs displayed both inside and outside prohibiting smoking and naked flames - Where appropriate, suitable cylinder trolleys to be used to transport cylinders, fixing the cylinder onto the trolley using the restraints provided. Trolleys to be visually inspected before use and any that are damaged or have defective/missing safety restraints to be removed from use until properly repaired by a competent person - When delivered to the delivery destination, the cylinder should either remain in its trolley with the restraints in place or transferred to a strapped storage location - Implement the safety precautions in the Manual Handling Checklist CONTINUED ON PAGE 2 |  |
| Who may be affected by the task/activity? (Tick all applicable boxes) |  |  |  |  |  |
| Compass employees/Agency staff |  |  |  |  |  |
| Customers / Client staff |  |  |  |  |  |
| Visitors / Members of the Public |  |  |  |  |  |
| Maintenance personnel |  |  |  |  |  |
| Delivery personnel |  |  |  |  |  |
| Other (specify below) |  |  |  |  |  |

| # Ref. POR 04 | Task/Activity: Medical Gases (Page 2 of 2) |  |  | Assessment date: |
| --- | --- | --- | --- | --- |
| Unit No: | Unit name/location: |  |  | Assessed by: |
| Hazard |  | Safety Risk | Safe System of Work / Control Measures |  |
|  |  |  | Where it is a site requirement to fit cylinders to manifolds, regulators or flow meters, the following additional control measures must be observed: - Cylinders should be used in date order, ensuring that they are in date - Only the correct regulator must be used for each gas cylinder i.e. do not use an air regulator with an oxygen cylinder. - Regulators and hoses to be checked for condition and leaks at each cylinder change - Any cylinder that shows signs of leakage (frosting around the valve or cylinder, or a “hissing sound”) should be isolated and the manager informed. - Only approved leak detection fluid must be used - Check that the seal is intact before removing. - Check for oil and grease or dirt in the valve. If any is present, the cylinder should not be used and reported to the manager for return - Check for oil and grease or dirt in the manifold, yoke and regulator. If any is present, do not fit but report to the manager for servicing. Check the “O” ring and if necessary, replace it - Fit the regulator, manifold or yoke to the cylinder using reasonable force. Do not use excessive force as this may damage the valve outlet threads - Open the cylinder valve slowly with a standard valve key, check for leaks, close the valve leaving the key in place |  |
| IMPORTANT - This risk assessment should be reviewed every 3 years, or whenever there is a significant change in the task or activity and following any accident or incident involving this task or activity. This risk assessment must be retained for a period of 6 years. |  |  |  |  |